RISCV-VDP-SoC

RISC-V Video Display Processor System-on-Chip

SYSTEM & MODULE FUNCTIONAL SPECIFICATION

| Document Item | Value |
| --- | --- |
| Document ID | RISCV-VDP-SPEC-001 |
| Revision | Rev 0.1 |
| Status | Engineering Draft / RTL Baseline |
| Target | ASIC/FPGA-style synthesizable RTL |
| Implementation | Xilinx Vivado |
| RTL | Verilog-2001 |
| Verification | Independent UVM-based DV |
| Primary Interfaces | RISC-V native mem_*, AXI4-Lite, VGA |

# 1. Document Control

| Revision | Date | Description | Status |
| --- | --- | --- | --- |
| 0.1 | 2026-07-30 | Initial system and module specification baseline | Draft |

## 1.1 Normative Language

SHALL/MUST define mandatory requirements. SHOULD defines a recommended engineering practice. MAY defines an optional choice. TBD identifies a value or behavior that must be frozen before final RTL sign-off.

## 1.2 Purpose and Scope

This document defines the functional, interface, microarchitectural, timing, reset, synthesis, lint, CDC, STA, and verification requirements for the RISCV-VDP-SoC. It is the implementation contract for RTL design and the functional source for the independent DV plan.

## 1.3 Design Constraints

Verilog-2001 synthesizable RTL.

Xilinx Vivado is the target implementation environment.

No vendor-specific primitives in functional RTL.

A separate UVM DV engineer verifies the completed SoC.

The actual selected PicoRV32 RTL is the source of truth for CPU behavior.

# 2. System Architecture

The SoC integrates a 32-bit RISC-V processor, memory, memory-mapped peripherals, an external AXI4-Lite control interface, and a video display path. The CPU uses the native PicoRV32 mem_* interface. The optional PicoRV32 AXI adapter is not assumed to be instantiated.

External AXI4-Lite Master
        |
        v
   AXI4-Lite Slave
        |
        v
  SoC Memory-Mapped Fabric
        |
  +-----+------+----------------+
  |            |                |
CPU native   Memory        Peripheral space
mem_*             |        +----+----+----+
                  |        |         |    |
                RAM/ROM  Registers Sensor RF
                                      |
                                  VGA/VDP

## 2.1 System Requirements

SR-001: The SoC SHALL provide a 32-bit memory-mapped control space.

SR-002: The SoC SHALL integrate the selected PicoRV32/picorv32a processor.

SR-003: The CPU SHALL use its native mem_* interface unless an explicit architecture revision selects the AXI adapter.

SR-004: The top-level control interface SHALL implement AXI4-Lite with 32-bit address and data paths.

SR-005: Sensor and RF status SHALL be software-readable.

SR-006: Display configuration SHALL control the video path.

SR-007: VGA timing and pixel generation SHALL be deterministic after reset.

SR-008: Complete RTL SHALL be synthesizable in Vivado.

# 3. Clock and Reset Architecture

| Item | Requirement |
| --- | --- |
| System clock | Primary synchronous clock; frequency TBD |
| Pixel clock | Separate pixel domain if required by selected VGA timing |
| Reset | Active-low synchronous project reset, rst_n |
| CDC | All crossings between clock domains SHALL use CDC-safe structures |
| Constraints | All clocks SHALL be explicitly constrained in Vivado |

# 4. System Address Map

| Offset | Register | Access | Description |
| --- | --- | --- | --- |
| 0x00 | VERSION | RO | IP version |
| 0x04 | CONTROL | RW | Enable, reset, mode |
| 0x08 | DISPLAY_CFG | RW | Theme, brightness |
| 0x0C | PATTERN_CFG | RW | Pattern selection |
| 0x10 | BATTERY_PERCENT | RO | Battery status |
| 0x14 | BATTERY_VOLTAGE | RO | Battery status |
| 0x18 | TEMPERATURE | RO | Temperature |
| 0x1C | RSSI | RO | RF telemetry |
| 0x20 | LINK_STATUS | RO | RF link status |
| 0x24 | ERROR_STATUS | RO | Reserved/status placeholder |
| 0x28 | INTERRUPT_STATUS | RO | Reserved |
| 0x2C | INTERRUPT_ENABLE | RW | Reserved/future use |

Register reset values, field widths, reserved-bit behavior, and exact encodings SHALL be frozen from final RTL. Unspecified fields are TBD.

# 5. RISC-V Processor — PicoRV32/picorv32a

Purpose: provide embedded 32-bit RISC-V execution for instruction fetch, integer computation, control flow, and memory-mapped load/store operations.

## 5.1 Selected Configuration

| Parameter | Value |
| --- | --- |
| ENABLE_REGS_16_31 | 1 |
| ENABLE_REGS_DUALPORT | 1 |
| COMPRESSED_ISA | 0 |
| ENABLE_MUL | 0 |
| ENABLE_FAST_MUL | 1 |
| ENABLE_DIV | 0 |
| ENABLE_PCPI | 0 |
| ENABLE_IRQ | 0 |
| ENABLE_IRQ_QREGS | 0 |
| ENABLE_IRQ_TIMER | 0 |
| ENABLE_COUNTERS | 1 |
| ENABLE_COUNTERS64 | 1 |
| BARREL_SHIFTER | 1 |
| TWO_STAGE_SHIFT | 1 |
| CATCH_MISALIGN | 1 |
| CATCH_ILLINSN | 1 |
| REGS_INIT_ZERO | 1 |
| PROGADDR_RESET | 0x00000000 |
| PROGADDR_IRQ | 0x00000010 |
| STACKADDR | 0x00000FFC |

## 5.2 CPU Port Specification

| Signal | Dir | Width | Description |
| --- | --- | --- | --- |
| clk | I | 1 | Processor clock |
| resetn | I | 1 | Active-low reset |
| trap | O | 1 | Trap indication |
| mem_valid | O | 1 | Memory transaction valid |
| mem_instr | O | 1 | Instruction access |
| mem_ready | I | 1 | Transaction completion |
| mem_addr | O | 32 | Address |
| mem_wdata | O | 32 | Write data |
| mem_wstrb | O | 4 | Byte strobes |
| mem_rdata | I | 32 | Read data |
| irq | I | 32 | Interrupt input; disabled |
| eoi | O | 32 | IRQ completion; disabled |

## 5.3 Functional Requirements

RV32I-oriented execution as supported by the selected RTL.

x0 SHALL remain zero.

Instruction fetch and data access SHALL use mem_valid/mem_ready protocol.

Load/store byte strobes SHALL be correct.

Branches and jumps SHALL update PC correctly.

Illegal instruction and misalignment detection SHALL follow enabled RTL behavior.

Fast multiplication is enabled; division and compressed ISA are disabled.

Reset PC is 0x00000000.

## 5.4 Timing, Reset, Synthesis, Lint, CDC, STA

Memory wait states are permitted; transaction information SHALL obey the actual PicoRV32 contract.

Reset behavior SHALL match the integration wrapper requirement.

No inferred latches or combinational feedback.

Width and signedness must be explicit.

CPU is single-clock unless integration creates another domain.

CPU and memory timing must be closed by Vivado STA.

## 5.5 DV Scenarios

Reset/first fetch

ALU and logical operations

Register x0 protection

Loads/stores

Branches/jumps

Illegal instruction

Misalignment

Fast multiply

Memory stalls

Long instruction sequences

# 6. Memory Subsystem and Interconnect

Routes CPU native mem_* transactions to instruction/data memory and memory-mapped peripheral targets. Address decoding, target selection, response routing, and any required arbitration SHALL be explicit.

## 6.1 Requirements

No overlapping decode regions.

Unmapped accesses have deterministic behavior.

Ordering follows the selected single-outstanding transaction architecture.

If CPU and AXI can access the same target concurrently, arbitration/collision behavior SHALL be explicitly defined before RTL sign-off.

No hidden coherency/cache behavior is assumed.

# 7. AXI4-Lite Slave / Control Interface

Purpose: provide the external 32-bit memory-mapped control interface. This interface is separate from the CPU native mem_* bus in the baseline architecture.

| Signal Group | Signals | Width |
| --- | --- | --- |
| Write address | AWADDR, AWVALID, AWREADY | 32/1/1 |
| Write data | WDATA, WSTRB, WVALID, WREADY | 32/4/1/1 |
| Write response | BRESP, BVALID, BREADY | 2/1/1 |
| Read address | ARADDR, ARVALID, ARREADY | 32/1/1 |
| Read data | RDATA, RRESP, RVALID, RREADY | 32/2/1/1 |

## 7.1 Protocol Requirements

AW and W channels SHALL be independently accepted.

One outstanding transaction is supported.

BVALID SHALL remain asserted until BREADY.

RVALID SHALL remain asserted until RREADY.

WSTRB SHALL control writable bytes.

Valid accesses return OKAY; invalid/unmapped accesses SHALL use the defined error policy.

Reset clears outstanding transaction/response state.

## 7.2 DV Scenarios

AW-before-W

W-before-AW

Backpressure

Back-to-back reads/writes

WSTRB

Unmapped address

Reset during idle/transaction

# 8. Register Bank

Implements software-visible RW configuration and RO status registers.

| Interface | Direction | Width | Purpose |
| --- | --- | --- | --- |
| clk/rst_n | I | 1/1 | Clock/reset |
| wr_en/wr_addr/wr_data/wr_strb | I | 1/32/32/4 | Write path |
| rd_en/rd_addr | I | 1/32 | Read path |
| rd_data | O | 32 | Read data |
| status inputs | I | TBD | Sensor/RF RO values |
| configuration outputs | O | TBD | CONTROL/DISPLAY/PATTERN fields |

Address decode SHALL be complete.

RO writes SHALL have no effect.

Reserved fields SHALL be deterministic.

WSTRB SHALL be honored.

Reset values SHALL be frozen before sign-off.

No inferred latches.

# 9. Sensor Status Peripheral

Digital status model providing battery percentage, battery voltage, and temperature. It models digital data only; no analog/ADC behavior is implied.

| Signal | Dir | Width | Description |
| --- | --- | --- | --- |
| clk | I | 1 | Clock |
| rst_n | I | 1 | Reset |
| battery_percent | I | TBD | Battery percentage |
| battery_voltage | I | TBD | Battery voltage |
| temperature | I | TBD | Temperature |

Widths, ranges, scaling, and update source are TBD until frozen.

Asynchronous inputs SHALL be synchronized.

Values SHALL be deterministic and within documented ranges.

## DV Scenarios

Reset

Minimum/maximum values

Updates

Out-of-range behavior

Register visibility

CDC if external inputs

# 10. RF Telemetry Peripheral

Digital telemetry model providing RSSI and link status.

| Signal | Dir | Width | Description |
| --- | --- | --- | --- |
| clk | I | 1 | Clock |
| rst_n | I | 1 | Reset |
| rssi | I | TBD | Signal strength |
| link_status | I | TBD | Link state |

RSSI encoding/range and link-status encoding are TBD until frozen.

No physical RF interface is modeled.

Asynchronous telemetry SHALL be synchronized.

# 11. VGA Timing Controller

Generates horizontal/vertical counters, sync signals, active-video indication, and pixel coordinates.

| Signal | Dir | Width | Description |
| --- | --- | --- | --- |
| pix_clk | I | 1 | Pixel clock |
| rst_n | I | 1 | Reset |
| enable | I | 1 | Display enable |
| hsync | O | 1 | Horizontal sync |
| vsync | O | 1 | Vertical sync |
| active_video | O | 1 | Active region |
| pixel_x | O | TBD | X coordinate |
| pixel_y | O | TBD | Y coordinate |

Resolution and timing constants are TBD until frozen.

Counters SHALL have deterministic rollover.

Sync polarity and porch definitions SHALL be explicit.

Separate pixel clock implies a CDC boundary.

## STA

The pixel clock SHALL be constrained separately where applicable. VGA timing paths are cycle-sensitive and require exact counter boundary analysis.

# 12. Video Pattern Generator / VDP

Generates RGB pixel values from VGA coordinates, pattern selection, display configuration, and selected status values.

| Signal | Dir | Width | Description |
| --- | --- | --- | --- |
| pix_clk | I | 1 | Pixel clock |
| rst_n | I | 1 | Reset |
| active_video | I | 1 | Active display |
| pixel_x/pixel_y | I | TBD | Coordinates |
| pattern_cfg | I | TBD | Pattern selection |
| display_cfg | I | TBD | Theme/brightness |
| status | I | Various | Sensor/RF values |
| red/green/blue | O | TBD | Pixel outputs |

RGB width is TBD.

Brightness arithmetic SHALL be width-safe and saturating if required.

Configuration/status crossing into pixel clock SHALL be CDC-safe.

Blanking SHALL produce deterministic output.

No font/bitmap engine is implied unless implemented.

# 13. Top-Level SoC Integration

Instantiates and connects CPU, memory, AXI4-Lite, register bank, sensor/RF peripherals, VGA timing, and video generation.

All clocks/resets SHALL be explicitly connected.

Every output SHALL have a single driver.

Address map SHALL be consistent across all decoders.

CPU native mem_* and external AXI4-Lite remain conceptually separate in the baseline.

Any shared-target CPU/AXI collision policy SHALL be explicit.

Vivado clock and I/O constraints are part of implementation, not this functional RTL specification.

# 14. Verification Requirements

Processor instruction/reference-model checking.

CPU memory protocol and wait-state verification.

AXI4-Lite protocol, backpressure, WSTRB, response and reset tests.

Register RO/RW/reset/partial-write tests.

Sensor/RF range and update tests.

VGA cycle-accurate timing tests.

VDP pattern/configuration tests.

End-to-end SoC CPU/AXI/peripheral/display scenarios.

No X/Z propagation on functional outputs.

# 15. Synthesis, Lint, CDC and STA

| Area | Requirement |
| --- | --- |
| Synthesis | Vivado synthesis SHALL complete without critical errors. |
| Lint | No critical latch, width, multiple-driver, undriven-net or clock/reset violations. |
| CDC | All asynchronous clock/data crossings SHALL be identified and safely transferred. |
| STA | All clocks SHALL be constrained and required timing targets met. |
| Resource | Actual utilization is TBD until Vivado synthesis. |

# 16. Acceptance Criteria

Complete RTL synthesizes in Vivado.

RISC-V program executes correctly.

AXI4-Lite register accesses are correct.

Sensor/RF status is readable.

Display configuration reaches VDP.

VGA timing is deterministic.

UVM verification passes required scenarios.

Actual timing/utilization/coverage results are recorded from tools; no fabricated values.

# 17. Traceability Matrix

| Requirement | Primary Module | Verification Intent |
| --- | --- | --- |
| CPU execution | RISC-V | Instruction/reference comparison |
| Memory access | CPU + Interconnect | Read/write/stall tests |
| AXI control | AXI Slave | Protocol tests |
| Configuration | Register Bank | RW/RO/WSTRB/reset |
| Battery/temp | Sensor | Range/update/readback |
| RF | RF Telemetry | Encoding/update/readback |
| Video timing | VGA Controller | Cycle-accurate timing |
| Pixel generation | VDP | Pattern/configuration |
| Integration | Top Level | End-to-end UVM |

# 18. Open Issues / TBD

| ID | Item | Closure |
| --- | --- | --- |
| TBD-001 | Exact CPU RTL revision/hash | Freeze source revision |
| TBD-002 | Register field widths/reset values | Freeze register spec |
| TBD-003 | Complete memory map | Freeze interconnect |
| TBD-004 | CPU/AXI simultaneous access policy | Define arbitration |
| TBD-005 | VGA resolution/timing | Freeze timing profile |
| TBD-006 | RGB width/brightness behavior | Freeze pixel interface |
| TBD-007 | Sensor widths/ranges/source | Freeze digital model |
| TBD-008 | RF encoding | Freeze telemetry interface |
| TBD-009 | Clock frequencies/XDC constraints | Create constraints |
| TBD-010 | Final Vivado/DV results | Run tools and record |

# 19. Known Limitations

No cache/coherency requirement.

Optional PicoRV32 AXI adapter is not assumed.

IRQ, division and compressed ISA are disabled in the selected CPU configuration.

Sensors/RF are digital models, not physical interfaces.

Exact VGA profile and dashboard rendering remain to be frozen.

Measured implementation results are intentionally not fabricated.

# 20. Final Sign-Off Checklist

Architecture approved.

CPU RTL revision frozen.

AXI ownership/separation confirmed.

Address map frozen.

Ports frozen.

Clock/reset plan frozen.

CDC paths reviewed.

Register fields/reset values frozen.

VGA profile frozen.

Lint clean.

Vivado synthesis successful.

STA constraints complete.

UVM plan derived from this specification.

All critical TBDs closed or waived.