RISCV-VDP-SoC

RISC-V Video Display Processor System-on-Chip

MINIMUM VIABLE PRODUCT (MVP) SYSTEM & MODULE SPECIFICATION

| Document Item | Value |
| --- | --- |
| Document ID | RISCV-VDP-SPEC-001 |
| Revision | 1.0 |
| Status | Baseline / Implementation Scope |
| Target | ASIC/FPGA-style synthesizable RTL |
| RTL Language | Verilog-2001 |
| Implementation | Xilinx Vivado |
| Verification | Independent UVM-based DV |
| Project Objective | Working RISC-V + AXI4-Lite controlled VDP SoC |

# 1. Document Control

| Revision | Date | Description | Status |
| --- | --- | --- | --- |
| 0.1 | 2026-07-30 | Initial broad architecture specification | Superseded |
| 1.0 | 2026-07-30 | MVP scope frozen for RTL implementation | Baseline |

## 1.1 Scope Freeze

Revision 1.0 is intentionally reduced from the broad architectural specification to a realistic implementation baseline. The objective is a demonstrable, synthesizable and verifiable SoC rather than implementation of every possible production feature.

## 1.2 Normative Language

SHALL/MUST indicates a mandatory requirement. SHOULD indicates a recommended implementation practice. MAY indicates an optional choice. TBD indicates a parameter that must be frozen before the corresponding RTL is finalized.

# 2. Project Objective

Develop a synthesizable RISC-V based Video Display Processor System-on-Chip in Verilog-2001, targeting Xilinx Vivado. The SoC shall demonstrate CPU execution, memory-mapped peripheral access, external AXI4-Lite control, sensor/RF status handling, and VGA video generation.

## 2.1 MVP Architecture

+----------------------+
                         | External AXI4-Lite   |
                         | Master               |
                         +----------+-----------+
                                    |
                              +-----v-----+
                              | AXI4-Lite |
                              | Slave     |
                              +-----+-----+
                                    |
                              +-----v-----+
                              | Register  |
                              | Bank      |
                              +--+--+--+--+
                                 |  |  |
                         +-------+  |  +--------+
                         |          |           |
                     Sensor         RF          VDP
                         |          |           |
                         +----------+-----+-----+
                                         |
                                  +------v------+
                                  | VGA Timing  |
                                  | + Pattern   |
                                  +------+------+
                                         |
                                        VGA

                  +----------------+
                  | RISC-V         |
                  | PicoRV32       |
                  +-------+--------+
                          |
                       mem_*
                          |
                  +-------v--------+
                  | Address Decoder|
                  +---+--------+---+
                      |        |
                     RAM    Peripherals

## 2.2 Architectural Principle

The RISC-V processor uses its native PicoRV32 mem_* interface. The external AXI4-Lite interface is a separate SoC control path. The optional AXI adapter inside PicoRV32 is NOT required by this MVP architecture.

## 2.3 MVP Requirements

SR-001: Integrate the selected RISC-V/PicoRV32 RTL without unnecessary modification.

SR-002: Provide synthesizable instruction/data memory.

SR-003: Decode CPU memory transactions into memory and peripheral regions.

SR-004: Implement an external 32-bit AXI4-Lite slave.

SR-005: Implement the defined configuration/status register map.

SR-006: Provide simple deterministic sensor and RF telemetry values.

SR-007: Generate deterministic VGA timing.

SR-008: Generate at least simple selectable video patterns.

SR-009: Integrate all MVP modules at top level.

SR-010: Ensure Vivado synthesis compatibility.

# 3. Implementation Scope

| Module | MVP Status | Scope |
| --- | --- | --- |
| - RISC-V Processor | REUSE | Use existing selected PicoRV32 RTL |
| - Instruction/Data Memory | IMPLEMENT | Simple synthesizable memory |
| - CPU Address Decoder | IMPLEMENT | Memory/peripheral decode |
| - AXI4-Lite Slave | IMPLEMENT | Single-outstanding AXI4-Lite |
| - Register Bank | IMPLEMENT | RW configuration + RO status |
| - Sensor Status | IMPLEMENT | Deterministic digital values |
| - RF Telemetry | IMPLEMENT | Deterministic RSSI/link status |
| - VGA Timing | IMPLEMENT | Fixed/parameterized standard timing |
| - Pattern Generator | IMPLEMENT | Simple selectable patterns |
| - Top-Level SoC | IMPLEMENT | Full integration |
| - Advanced graphics | DEFERRED | Not required for Rev 1.0 |
| - Cache/coherency | DEFERRED | Out of scope |
| - Interrupt subsystem | DEFERRED | Not required for MVP |
| - Complex arbitration | DEFERRED | Avoided unless architecture requires it |

# 4. Clock and Reset

| Item | MVP Requirement |
| --- | --- |
| System clock | Single synchronous system clock; frequency set by Vivado constraints |
| Pixel clock | May be same clock for initial MVP or separate if required by VGA implementation |
| Reset | Active-low synchronous rst_n at project integration boundary |
| CDC | Required whenever separate system/pixel clocks are used |
| Reset state | All state and externally visible outputs shall have deterministic values |

The selected processor's native reset requirements shall be isolated in its integration wrapper if they differ from the project-level reset convention.

# 5. RISC-V Processor

The processor is the existing selected PicoRV32/picorv32a RTL. It is reused as the CPU execution engine and is not rewritten merely to add AXI4-Lite.

## 5.1 Selected Configuration

| Parameter | Selected Value |
| --- | --- |
| ENABLE_REGS_16_31 | 1 |
| ENABLE_REGS_DUALPORT | 1 |
| COMPRESSED_ISA | 0 |
| ENABLE_MUL | 0 |
| ENABLE_FAST_MUL | 1 |
| ENABLE_DIV | 0 |
| ENABLE_PCPI | 0 |
| ENABLE_IRQ | 0 |
| ENABLE_IRQ_QREGS | 0 |
| ENABLE_IRQ_TIMER | 0 |
| ENABLE_COUNTERS | 1 |
| ENABLE_COUNTERS64 | 1 |
| BARREL_SHIFTER | 1 |
| TWO_STAGE_SHIFT | 1 |
| CATCH_MISALIGN | 1 |
| CATCH_ILLINSN | 1 |
| REGS_INIT_ZERO | 1 |
| PROGADDR_RESET | 0x00000000 |
| PROGADDR_IRQ | 0x00000010 |
| STACKADDR | 0x00000FFC |

## 5.2 Native CPU Interface

| Signal | Dir | Width | Description |
| --- | --- | --- | --- |
| clk | I | 1 | CPU clock |
| resetn | I | 1 | CPU reset |
| mem_valid | O | 1 | Transaction valid |
| mem_instr | O | 1 | Instruction access |
| mem_ready | I | 1 | Transaction complete |
| mem_addr | O | 32 | Address |
| mem_wdata | O | 32 | Write data |
| mem_wstrb | O | 4 | Byte write strobes |
| mem_rdata | I | 32 | Read data |
| trap | O | 1 | Trap indication |

## 5.3 CPU Requirements

CPU SHALL execute the instruction set supported by the selected RTL.

x0 SHALL remain architecturally zero.

Memory requests SHALL be held according to the actual PicoRV32 mem_* protocol until completion.

Reset PC SHALL be 0x00000000.

Fast multiplication is enabled; division and compressed ISA are disabled.

Interrupt functionality is not required for MVP.

## 5.4 CPU Verification Intent

Reset and first instruction fetch

ALU/logical operations

Register operations

Loads/stores

Branches/jumps

Multiply

Illegal instruction

Misaligned access

Memory wait states

# 6. Memory and CPU Interconnect

A simple address decoder shall route CPU mem_* transactions to instruction/data memory or the memory-mapped peripheral region.

| Region | Address Range | Purpose |
| --- | --- | --- |
| Program/Data Memory | TBD; implementation-defined | CPU instruction/data storage |
| Peripheral Space | Base TBD | Memory-mapped SoC peripherals |
| Unmapped | All other addresses | Deterministic default/error behavior |

No overlapping decode regions.

Read and write responses shall be deterministic.

No cache or coherency is required.

Avoid unnecessary arbitration in the MVP.

# 7. AXI4-Lite Slave

The AXI4-Lite slave provides external software/host/DV access to the SoC register space. It is separate from the CPU native mem_* interface.

| Channel | Signals | Width |
| --- | --- | --- |
| Write Address | AWADDR, AWVALID, AWREADY | 32/1/1 |
| Write Data | WDATA, WSTRB, WVALID, WREADY | 32/4/1/1 |
| Write Response | BRESP, BVALID, BREADY | 2/1/1 |
| Read Address | ARADDR, ARVALID, ARREADY | 32/1/1 |
| Read Data | RDATA, RRESP, RVALID, RREADY | 32/2/1/1 |

One outstanding transaction SHALL be supported.

AW and W SHALL be accepted independently.

BVALID SHALL remain asserted until BREADY.

RVALID SHALL remain asserted until RREADY.

WSTRB SHALL control byte writes.

AXI4-Lite only; no bursts.

## 7.1 AXI Verification Intent

AW-before-W

W-before-AW

Backpressure

Back-to-back accesses

WSTRB

Read/write response holding

Unmapped access

Reset

# 8. Register Bank

The register bank stores software-controlled configuration and exposes read-only sensor/RF status.

| Offset | Register | Access | MVP Function |
| --- | --- | --- | --- |
| 0x00 | VERSION | RO | IP version constant |
| 0x04 | CONTROL | RW | Enable/mode control |
| 0x08 | DISPLAY_CFG | RW | Theme/brightness |
| 0x0C | PATTERN_CFG | RW | Pattern selection |
| 0x10 | BATTERY_PERCENT | RO | Battery status |
| 0x14 | BATTERY_VOLTAGE | RO | Battery voltage |
| 0x18 | TEMPERATURE | RO | Temperature |
| 0x1C | RSSI | RO | RF signal strength |
| 0x20 | LINK_STATUS | RO | RF link status |
| 0x24 | ERROR_STATUS | RO | Reserved; deterministic |
| 0x28 | INTERRUPT_STATUS | RO | Reserved; deterministic |
| 0x2C | INTERRUPT_ENABLE | RW | Reserved/future |

RO registers SHALL ignore writes.

WSTRB SHALL be honored for RW registers.

Reserved fields SHALL have deterministic behavior.

Reset values SHALL be explicitly defined in RTL.

# 9. Sensor Status Peripheral

The MVP sensor block is a digital status model. It does not model an analog sensor or ADC.

| Value | Width | MVP Behavior |
| --- | --- | --- |
| Battery percentage | TBD | Deterministic value or controlled test input |
| Battery voltage | TBD | Deterministic value or controlled test input |
| Temperature | TBD | Deterministic value or controlled test input |

Values SHALL be software-readable through the register bank.

Ranges and scaling SHALL be documented in the RTL.

Asynchronous external sources require synchronization.

# 10. RF Telemetry Peripheral

The MVP RF block is a digital telemetry model.

| Value | Width | MVP Behavior |
| --- | --- | --- |
| RSSI | TBD | Deterministic telemetry value |
| LINK_STATUS | TBD | Defined binary/status encoding |

No RF physical interface is modeled.

Encoding shall be deterministic.

Asynchronous sources require synchronization.

# 11. VGA Timing Controller

Generate horizontal/vertical timing, sync, active-video, and pixel coordinates. A fixed standard timing profile is acceptable for MVP.

| Signal | Dir | Width | Description |
| --- | --- | --- | --- |
| pix_clk | I | 1 | Pixel clock |
| rst_n | I | 1 | Reset |
| enable | I | 1 | Display enable |
| hsync | O | 1 | Horizontal sync |
| vsync | O | 1 | Vertical sync |
| active_video | O | 1 | Active region |
| pixel_x | O | TBD | X coordinate |
| pixel_y | O | TBD | Y coordinate |

Counter rollover SHALL be exact.

Sync polarity SHALL be explicit.

Blanking and active intervals SHALL be deterministic.

If a separate pixel clock is used, CDC SHALL be addressed.

# 12. Video Pattern Generator / VDP

Generate simple RGB output based on coordinates, PATTERN_CFG, DISPLAY_CFG and selected status values.

| Input/Output | Width | Purpose |
| --- | --- | --- |
| pixel_x/pixel_y | TBD | Pixel coordinates |
| pattern_cfg | TBD | Pattern selection |
| display_cfg | TBD | Theme/brightness |
| status values | Various | Dashboard data |
| RGB | TBD | Video output |

MVP patterns may be simple deterministic patterns such as solid fill, color bars, checkerboard, borders, or a basic status/dashboard representation. Complex fonts, sprites, and graphics engines are deferred.

Outputs shall blank deterministically outside active video.

Brightness arithmetic shall avoid overflow.

Configuration crossing into the pixel domain SHALL be CDC-safe if clocks differ.

# 13. Top-Level Integration

The top-level shall instantiate the CPU, memory, address decoder, AXI4-Lite slave, register bank, sensor/RF models, VGA timing controller, and pattern generator.

CPU --> mem_* --> Address Decoder --> Memory / Peripheral Registers
                                           |
AXI4-Lite --> AXI Slave ------------------+
                                           |
                                Sensor / RF / VDP
                                           |
                                      VGA Outputs

CPU native mem_* and AXI4-Lite are separate interfaces.

All modules SHALL have explicit clock/reset connections.

All outputs SHALL have a single driver.

Address-map definitions SHALL be consistent.

Top-level shall be synthesizable in Vivado.

# 14. RTL Coding Standard

Verilog-2001 only.

One module per source file.

Meaningful signal and parameter names.

Active-low synchronous reset for project RTL.

No inferred latches.

No multiple drivers.

No combinational feedback.

Explicit default assignments in combinational logic.

Explicit widths and signedness where arithmetic is used.

No vendor-specific primitives in functional RTL.

Parameterize timing/memory dimensions where practical.

Keep combinational paths short enough for timing closure.

# 15. RTL Design Review Checklist

Specification-to-RTL interface match.

Reset behavior reviewed.

Every register has a defined reset value or documented exception.

Every combinational process is fully assigned.

All address comparisons are width-correct.

AXI AW/W independence verified by inspection.

WSTRB behavior verified.

CPU mem_* handshake verified against the selected PicoRV32 source.

CDC paths identified.

Unused/reserved outputs have deterministic values.

No accidental inferred memories/latches.

# 16. Verification Scope for Independent DV

The DV engineer shall independently create the UVM environment. This document defines only the functional intent.

CPU execution and memory interface.

AXI4-Lite protocol and register accesses.

Register reset, RO/RW, WSTRB and reserved fields.

Sensor/RF status readback and boundary values.

VGA timing and counter boundaries.

Pattern selection and display configuration.

End-to-end CPU-to-peripheral operation.

End-to-end AXI-to-peripheral operation.

Reset and recovery.

Negative/unmapped accesses.

# 17. Synthesis / Lint / CDC / STA

| Check | MVP Requirement |
| --- | --- |
| Vivado Synthesis | No critical synthesis errors. |
| Lint / SpyGlass | No critical latch, width, driver, reset or clocking violations. |
| CDC | All asynchronous crossings identified and safely handled. |
| STA | All clocks constrained; required paths meet the project timing target. |
| Implementation | No critical implementation errors; generated outputs are functionally connected. |
| Resource | Actual LUT/FF/BRAM/timing numbers recorded from Vivado; never fabricated. |

# 18. MVP Acceptance Criteria

AC-001: RISC-V RTL synthesizes and executes a test program.

AC-002: CPU can read/write memory-mapped registers.

AC-003: External AXI4-Lite can read/write the register bank.

AC-004: Sensor and RF status are visible through the address map.

AC-005: PATTERN_CFG changes the generated pattern.

AC-006: VGA timing generates stable sync and active-video signals.

AC-007: Complete SoC synthesizes in Vivado.

AC-008: Independent DV demonstrates functional correctness for the defined scenarios.

# 19. Deferred Features

Complex dashboard graphics and text/font rendering.

Cache/coherency.

Interrupt controller and software interrupt handling.

AXI bursts or AXI4 full protocol.

Advanced multi-master arbitration.

Physical ADC/RF interfaces.

Power-management features.

Performance optimization beyond basic timing closure.

Deferred features SHALL NOT block Rev 1.0 MVP completion unless they become necessary for the selected demo or integration.

# 20. 2–3 Day Implementation Plan

| Stage | Primary Deliverables |
| --- | --- |
| Day 1 — Foundation | Integrate RISC-V, memory, CPU address decoder, register bank, basic simulation. |
| Day 1/2 — AXI | Implement AXI4-Lite slave and verify register read/write path. |
| Day 2 — Peripherals | Sensor, RF, VGA timing, simple pattern generator. |
| Day 2/3 — Integration | Top-level SoC, CPU program, AXI access, end-to-end simulation. |
| Day 3 — FPGA/Sign-off | Vivado synthesis/implementation, timing/resource reports, waveforms and final DV regression. |

The implementation plan is intentionally aggressive. If schedule pressure occurs, preserve the CPU + memory + register bank + AXI4-Lite path first, then add VGA and simplified peripherals.

# 21. Open Issues / Freeze Before RTL Sign-Off

| ID | Item | Required Closure |
| --- | --- | --- |
| TBD-001 | Exact PicoRV32 source revision/hash | Record RTL revision |
| TBD-002 | Program/data memory size | Freeze parameter |
| TBD-003 | Peripheral base address | Freeze address map |
| TBD-004 | Register reset values/field widths | Freeze register spec |
| TBD-005 | Sensor/RF widths and ranges | Freeze encodings |
| TBD-006 | VGA resolution/pixel timing | Freeze timing profile |
| TBD-007 | RGB width | Freeze video interface |
| TBD-008 | Clock frequencies | Freeze Vivado constraints |

# 22. Final Sign-Off

Architecture frozen at Rev 1.0 MVP scope.

Existing RISC-V RTL reused rather than unnecessarily rewritten.

CPU native mem_* path and external AXI4-Lite path remain distinct.

Address/register map frozen.

All module interfaces reviewed.

Reset and clock strategy reviewed.

CDC reviewed where applicable.

RTL linted and synthesized.

STA completed.

Independent DV regression completed for MVP requirements.

Measured implementation results documented.